-82-

## ANNEXURE – XIV

PRODUCT : DEHYDRATED ONION FLAKES

- DIMENSIONS OF EMPTY UNIT POUCHES (EXTERNAL)

| CAPACITY | LENGTH OF THE POUCH (MM) | WIDTH OF THE POUCH (MM) | SEAL WIDTH (MM) |
| --- | --- | --- | --- |
| 50 GRAMS | 180 | 130 | 10 |
| 100 GRAMS | 250 | 155 | 10 |

- DIMENSIONS OF FILLED UNIT POUCHES (EXTERNAL)

| CAPACITY | LENGTH OF THE POUCH (MM) | WIDTH OF THE POUCH (MM) | SEAL WIDTH (MM) |
| --- | --- | --- | --- |
| 50 GRAMS | 170 | 125 | 10 |
| 100 GRAMS | 240 | 150 | 10 |

- 83 –

## ANNEXURE – XV

- UNIT PACK CAPACITY – 50 GRAMS

BULK PACK CAPACITY – 4 KG

### NO. OF UNIT POUCHES / BULK BOX – 80 POUCHES

| SR. NO. | ARRANGEMENT OF POUCHES | INTERNAL DIMENSIONS OF THE BOX (MM) | BOARD AREA (SQ.CM) |
| --- | --- | --- | --- |
| 1 | 4 x 4 x 5 | 680 x 500 x 200 | 16870 |
| 2 | 5 x 4 x 4 | 850 x 500 x 160 | 18150 |
| 3 | 4 x 5 x 4 | 680 x 625 x 160 | 20881 |
| 4 | 5 x 2 x 8 | 850 x 250 x 320 | 12825 |
| 5 | 2 x 5 x 8 | 340 x 625 x 320 | 13068 |

#### UNIT PACK CAPACITY – 50 GRAMS

BULK PACK CAPACITY – 6 KG

### NO. OF UNIT POUCHES / BULK BOX – 120 POUCHES

| SR. NO. | ARRANGEMENT OF POUCHES | INTERNAL DIMENSIONS OF THE BOX (MM) | BOARD AREA (SQ.CM) |
| --- | --- | --- | --- |
| 1 | 4 x 3 x 10 | 680 x 375 x 400 | 14027.50 |
| 2 | 3 x 4 x 10 | 510 x 500 x 400 | 18630.00 |
| 3 | 6 x 2 x 10 | 1020 x 250 x 400 | 15210.00 |
| 4 | 2 x 6 x 10 | 340 x 750 x 400 | 16502.00 |
| 5 | 3 x 5 x 8 | 510 x 625 x 320 | 19256.00 |
| 6 | 5 x 3 x 8 | 850 x 375 x 320 | 17375.00 |

- 84 -

#### UNIT PACK CAPACITY – 100 GRAMS

BULK PACK CAPACITY – 4 KG

### NO. OF UNIT POUCHES / BULK BOX – 40 POUCHES

| SR. NO. | ARRANGEMENT OF POUCHES | INTERNAL DIMENSIONS OF THE BOX (MM) | BOARD AREA (SQ.CM) |
| --- | --- | --- | --- |
| 1 | 5 x 4 x 2 | 1200 x 600 x 80 | 24820.00 |
| 2 | 4 x 5 x 2 | 960 x 750 x 80 | 28801.00 |
| 3 | 2 x 5 x 4 | 480 x 750 x 160 | 16064.00 |
| 4 | 5 x 2 x 4 | 1200 x 300 x 160 | 14030.00 |
| 5 | 4 x 2 x 5 | 960 x 300 x 200 | 11350.00 |
| 6 | 2 x 4 x 5 | 480 x 600 x 200 | 15028.00 |

- 85 -

#### UNIT PACK CAPACITY – 100 GRAMS

BULK PACK CAPACITY – 6 KG

### NO. OF UNIT POUCHES / BULK BOX – 60 POUCHES

| SR. NO. | ARRANGEMENT OF POUCHES | INTERNAL DIMENSIONS OF THE BOX (MM) | BOARD AREA (SQ.CM) |
| --- | --- | --- | --- |
| 1 | 6 x 5 x 2 | 1440 x 750 x 80 | 36769.00 |
| 2 | 5 x 6 x 2 | 1200 x 900 x 80 | 41650.00 |
| 3 | 5 x 4 x 3 | 1200 x 600 x 120 | 26280.00 |
| 4 | 4 x 5 x 3 | 960 x 750 x 120 | 30189.00 |
| 5 | 3 x 5 x 4 | 720 x 750 x 120 | 26312.00 |
| 6 | 5 x 2 x 6 | 1200 x 300 x 240 | 16470.00 |
| 7 | 5 x 3 x 4 | 1200 x 450 x 160 | 20435.00 |
| 8 | 2 x 5 x 6 | 480 x 750 x 240 | 18072.00 |

- 86 -

## ANNEXURE – XVI

##### GARLIC POWDER

- DIMENSIONS OF EMPTY UNIT POUCHES (EXTERNAL)

| CAPACITY | LENGTH OF THE POUCH (MM) | WIDTH OF THE POUCH (MM) | SEAL WIDTH (MM) |
| --- | --- | --- | --- |
| 100 GRAMS | 150 | 130 | 10 |
| 200 GRAMS | 170 | 140 | 10 |

- DIMENSIONS OF FILLED UNIT POUCHES (EXTERNAL)

| CAPACITY | LENGTH OF THE POUCH (MM) | WIDTH OF THE POUCH (MM) | SEAL WIDTH (MM) |
| --- | --- | --- | --- |
| 100 GRAMS | 140 | 125 | 10 |
| 200 GRAMS | 160 | 135 | 10 |

-87-

## ANNEXURE – XVII

PRODUCT : DEHYDRATED GARLIC POWDER

#### A.	UNIT PACK CAPACITY – 100 GRAMS

BULK PACK CAPACITY – 6 KG

### NO. OF UNIT POUCHES / BULK BOX – 60 POUCHES

| SR. NO. | ARRANGEMENT OF POUCHES | INTERNAL DIMENSIONS OF THE BOX (MM) | BOARD AREA (SQ.CM) |
| --- | --- | --- | --- |
| 1 | 6 x 5 x 2 | 840 x 625 x 40 | 19817.00 |
| 2 | 5 x 6 x 2 | 700 x 750 x 40 | 23305.00 |
| 3 | 5 x 4 x 3 | 700 x 500 x 60 | 13720.00 |
| 4 | 4 x 5 x 3 | 560 x 625 x 60 | 15004.00 |
| 5 | 3 x 5 x 4 | 420 x 625 x 80 | 10700.00 |
| 6 | 5 x 2 x 6 | 700 x 250 x 720 | 7215.00 |
| 7 | 5 x 3 x 4 | 700 x 375 x 80 | 10010.00 |
| 8 | 2 x 5 x 6 | 280 x 625 x 120 | 13857.00 |

-88-

#### UNIT PACK CAPACITY – 100 GRAMS

BULK PACK CAPACITY – 8 KG

### NO. OF UNIT POUCHES / BULK BOX – 80 POUCHES

| SR. NO. | ARRANGEMENT OF POUCHES | INTERNAL DIMENSIONS OF THE BOX (MM) | BOARD AREA (SQ.CM) |
| --- | --- | --- | --- |
| 1 | 4 x 4 x 5 | 560 x 500 x 100 | 13020.00 |
| 2 | 5 x 4 x 4 | 700 x 500 x 80 | 14210.00 |
| 3 | 4 x 5 x 4 | 560 x 625 x 80 | 14036.00 |
| 4 | 5 x 2 x 8 | 700 x 250 x 160 | 7995.00 |
| 5 | 2 x 5 x 8 | 280 x 625 x 160 | 14601.00 |

#### C.	UNIT PACK CAPACITY – 200 GRAMS

BULK PACK CAPACITY – 6 KG

### NO. OF UNIT POUCHES / BULK BOX – 30 POUCHES

| SR. NO. | ARRANGEMENT OF POUCHES | INTERNAL DIMENSIONS OF THE BOX (MM) | BOARD AREA (SQ.CM) |
| --- | --- | --- | --- |
| 1 | 5 x 3 x 2 | 800 x 405 x 50 | 11193.00 |
| 2 | 3 x 5 x 2 | 480 x 675 x 50 | 12510.00 |
| 3 | 2 x 5 x 3 | 320 x 675 x 75 | 8058.00 |
| 4 | 5 x 2 x 3 | 800 x 270 x 75 | 7555.00 |
| 5 | 2 x 3 x 5 | 320 x 405 x 125 | 6675.00 |
| 6 | 3 x 2 x 5 | 480 x 270 x 125 | 6122.00 |

-89-

#### D.	UNIT PACK CAPACITY – 200 GRAMS

BULK PACK CAPACITY – 8 KG

### NO. OF UNIT POUCHES / BULK BOX – 40 POUCHES

| SR. NO. | ARRANGEMENT OF POUCHES | INTERNAL DIMENSIONS OF THE BOX (MM) | BOARD AREA (SQ.CM) |
| --- | --- | --- | --- |
| 1 | 5 x 4 x 2 | 800 x 540 x 50 | 16107.00 |
| 2 | 4 x 5 x 2 | 640 x 675 x 50 | 18492.00 |
| 3 | 2 x 5 x 4 | 320 x 675 x 100 | 8568.00 |
| 4 | 5 x 2 x 4 | 800 x 270 x 100 | 8103.00 |
| 5 | 4 x 2 x 5 | 640 x 270 x 125 | 7386.50 |
| 6 | 2 x 4 x 5 | 320 x 540 x 125 | 7876.50 |

-90-

## ANNEXURE –XVIII

PRODUCT : ACCELERATED FREEZE DRIED CORN

#### A.     DIMENSIONS OF EMPTY UNIT POUCHES (EXTERNAL)

| CAPACITY | LENGTH OF THE POUCH (MM) | WIDTH OF THE POUCH (MM) | SEAL WIDTH (MM) |
| --- | --- | --- | --- |
| 50 GRAMS | 160 | 135 | 10 |
| 100 GRAMS | 200 | 145 | 10 |

#### DIMENSIONS OF FILLED UNIT POUCHES (EXTERNAL)

| CAPACITY | LENGTH OF THE POUCH (MM) | WIDTH OF THE POUCH (MM) | SEAL WIDTH (MM) |
| --- | --- | --- | --- |
| 50 GRAMS | 150 | 130 | 10 |
| 100 GRAMS | 190 | 140 | 10 |

- 91 -

## ANNEXURE – XIX

#### UNIT PACK CAPACITY – 50 GRAMS

BULK PACK CAPACITY – 4 KG

### NO. OF UNIT POUCHES / BULK BOX – 80 POUCHES

| SR. NO. | ARRANGEMENT OF POUCHES | INTERNAL DIMENSIONS OF THE BOX (MM) | BOARD AREA (SQ.CM) |
| --- | --- | --- | --- |
| 1 | 4 x 4 x 5 | 600 x 520 x 75 | 13625.50 |
| 2 | 5 x 4 x 4 | 750 x 520 x 60 | 15022.00 |
| 3 | 4 x 5 x 4 | 600 x 650 x 60 | 13845.00 |
| 4 | 5 x 2 x 8 | 750 x 260 x 120 | 7866.00 |
| 5 | 2 x 5 x 8 | 300 x 650 x 120 | 15015.00 |

#### UNIT PACK CAPACITY – 50 GRAMS

BULK PACK CAPACITY – 6 KG

### NO. OF UNIT POUCHES / BULK BOX – 120 POUCHES

| SR. NO. | ARRANGEMENT OF POUCHES | INTERNAL DIMENSIONS OF THE BOX (MM) | BOARD AREA (SQ.CM) |
| --- | --- | --- | --- |
| 1 | 4 x 3 x 10 | 600 x 390 x 150 | 10962.00 |
| 2 | 3 x 4 x 10 | 450 x 520 x 150 | 13333.00 |
| 3 | 6 x 2 x 10 | 900 x 260 x 150 | 9717.00 |
| 4 | 2 x 6 x 10 | 300 x 780 x 150 | 20553.00 |
| 5 | 3 x 5 x 8 | 450 x 650 x 120 | 17325.00 |
| 6 | 5 x 3 x 8 | 750 x 390 x 120 | 11883.00 |

- 92 -

#### UNIT PACK CAPACITY – 100 GRAMS

BULK PACK CAPACITY – 4 KG

### NO. OF UNIT POUCHES / BULK BOX – 40 POUCHES

| SR. NO. | ARRANGEMENT OF POUCHES | INTERNAL DIMENSIONS OF THE BOX (MM) | BOARD AREA (SQ.CM) |
| --- | --- | --- | --- |
| 1 | 5 x 4 x 2 | 950 x 560 x 40 | 18420.00 |
| 2 | 4 x 5 x 2 | 760 x 700 x 40 | 21978.00 |
| 3 | 2 x 5 x 4 | 380 x 700 x 80 | 10166.00 |
| 4 | 5 x 2 x 4 | 950 x 280 x 80 | 9036.00 |
| 5 | 4 x 2 x 5 | 760 x 280 x 100 | 8094.00 |
| 6 | 2 x 4 x 5 | 380 x 700 x 100 | 10608.00 |

- 93 -

#### UNIT PACK CAPACITY – 100 GRAMS

BULK PACK CAPACITY – 6 KG

### NO. OF UNIT POUCHES / BULK BOX – 60 POUCHES

| SR. NO. | ARRANGEMENT OF POUCHES | INTERNAL DIMENSIONS OF THE BOX (MM) | BOARD AREA (SQ.CM) |
| --- | --- | --- | --- |
| 1 | 6 x 5 x 2 | 1140 x 700 x 40 | 27602.00 |
| 2 | 5 x 6 x 2 | 950 x 840 x 40 | 31944.00 |
| 3 | 5 x 4 x 3 | 950 x 560 x 60 | 19034.00 |
| 4 | 4 x 5 x 3 | 760 x 700 x 60 | 22572.00 |
| 5 | 3 x 5 x 4 | 570 x 700 x 80 | 16835.00 |
| 6 | 5 x 2 x 6 | 950 x 280 x 120 | 10040.00 |
| 7 | 5 x 3 x 4 | 950 x 420 x 80 | 13950.00 |
| 8 | 2 x 5 x 6 | 380 x 700 x 120 | 11050.00 |

- 94 -

ANNEXURE - XX

EXTERNAL DIMENSIONS OF RECOMMENDED BULK PACKS

FOR DEHYDRATED ONION FLAKES

| DIMESNIONS | 50 GRAMS UNIT PACK |  | 100 GRAMS UNIT PACK |  |
| --- | --- | --- | --- | --- |
|  | 4 KG | 6 KG | 4 KG | 6 KG |
| LENGTH (MM) | 635 | 690 | 610 | 760 |
| WIDTH (MM) | 350 | 385 | 490 | 490 |
| HEIGHT (MM) | 340 | 420 | 220 | 260 |

ANNEXURE - XXI

EXTERNAL DIMENSIONS OF RECOMMENDED BULK PACKS

FOR DEHYDRATED GARLIC POWDER

| DIMESNIONS | 100 GRAMS UNIT PACK |  | 200 GRAMS UNIT PACK |  |
| --- | --- | --- | --- | --- |
|  | 6 KG | 8 KG | 6 KG | 8 KG |
| LENGTH (MM) | 635 | 570 | 415 | 550 |
| WIDTH (MM) | 430 | 510 | 330 | 330 |
| HEIGHT (MM) | 100 | 120 | 145 | 145 |

- 95 -

ANNEXURE - XXII

EXTERNAL DIMENSIONS OF RECOMMENDED BULK PACKS

FOR ACCELERATEDFREEZE DRIED CORN

| DIMESNIONS | 50 GRAMS UNIT PACK |  | 100 GRAMS UNIT PACK |  |
| --- | --- | --- | --- | --- |
|  | 4 KG | 6 KG | 4 KG | 6 KG |
| LENGTH (MM) | 610 | 610 | 710 | 710 |
| WIDTH (MM) | 530 | 400 | 390 | 390 |
| HEIGHT (MM) | 95 | 170 | 100 | 140 |

-96-

ANNEXURE – XXIII

SPECIFICATIONS DETAILS

PRODUCT : DEHYDRATED ONION FLAKES

UNIT POUCHES AND BULK PACK FOR UNIT POUCHES

I.	UNIT POUCH

MATERIAL				THICKNESS

1.	PET / AL. FOIL / LDPE	:	       12         /     9            /     37.5

LAMINATE                                        MICRON  /  MICRON /  MICRON

2.	MET-PET/LDPE		:	    12          /    37.5

MICRON  /    MICRON

The laminates shall be either extrusion laminated or in case of adhesive lamination, food grade odour free adhesives should be used

DIMENSIONS  (EXTERNAL)

(mm)

50 GRAMS			:	180 X 130 X 10

100 GRAMS			:	250 X 155 X 10

Performance:

Bond Strength (g/15 mm width)	 Min.

(In case of laminates only)				:	250.00

Heat Seal Strength (kg/15 mm width) Min.		:	2.00

Seal width (mm/Min.) 					:	5.00

Oxygen Transmission Rate (cc/m2/24 Hrs.) Max.

At 1 atm pressure 						:	3.00

Water Vapour Transmission Rate (g/m2/24 Hrs.)Max.

At 38  1C & 90  2% RH				:	1.00

- 96 A -

Note:

- While the above are indicative types of laminates that are recommended, any other flexible laminates or films comparable to the barrier and strength properties of the above types of laminates would also be acceptable.

2.       The laminate/co-extruded film could be transparent/ opaque/

coloured as per the requirement of the buyer

3.   	All the materials used shall be of food grade quality and pass the prescribed Indian Standards/Standards if any and as demanded by     the buyers.

- Labelling and printing requirements shall conform to as prescribed by the buyer.

5.	General requirements:

- The pouches shall be clean and without specs.

- The pouches shall be three sides or four sides sealed.

- The seal width shall be minimum 5 mm, particularly the

top seal. (For filled pouches)

- The pouches shall be either reverse or surface printed. In

the case of surface print, the print shall be compatible with the product and shall not smudge. The print shall be legible and easy to read.

- The pouch shall not fail either on the body or along the

seals when subjected to normal drop tests.

- All the pouches should be flushed with Nitrogen gas at

the time of  sealing to retain the freshness of the product.

-97-

# II.	BULK BOX

BOX FOR 4 KG AND 6 KG.

MATERIAL OF CONSTRUCTION	:	CORRUGATED FIBRE BOARD

SYLE OF BOX 	:	RSC

NO. OF PLIES	:	FIVE

TYPE OF FLUTE	:	B/B (NARROW/NARROW)

POSITION OF FLUTES	:	VERTICAL

DIMENSIONS (EXTERNAL) MM

4 KG BULK, 50 GMS UNIT PACK	:	635 X 350 X 340

6 KG BULK, 50 GMS UNIT PACK	:	690 X 385 X 420

4 KG BULK, 100 GMS UNIT PACK	:	610 X 490 X 220

6 KG BULK, 100 GMS UNIT PACK	:	760 X 490 X 260

GRAMMAGE (g/m2)	:	200/140/140/140/140

###### BURSTING STRENGTH (kg/cm2)		:	MINIMUM 14.00

COMPRESSION STRENGTH (kg)	:	MINIMUM 550.00

COBB VALUE (g/m2) (30 Min.)	:	MAXIMUM 155.00

NO. OF PIECES PER BOX	:	NOT MORE THAN TWO

MANUFACTURERS JOINT	:	BY STAPLE PINS

OTHER DETAILS AS PER IS:2771-1977 (PART-I)

-98-

SEPARATOR PLATES

MATERIAL OF CONSTRUCTION	:	CORRUGATED FIBRE BOARD

NO. OF PLIES	:	THREE

TYPE OF FLUTE	:	B (NARROW)

GRAMMAGE (g/m2)	:	100/100/100

BURSTING STRENGTH (kg/cm2)	:	MINIMUM 5.00

## PRESSURE SENSITIVE TAPE

MATERIAL	:	BOPP OR PVC

THICKNESS 	:	20 MICRONS

WIDTH	:	50 MM

ADHESIVE PROPERTY TO PASS AS PER IS:3676-1986

FOR FURTHER SPECIFICATION DETAILS REFER IS: 2880-1978

## REINFORCEMENT STRAP

MATERIAL OF CONSTRUCTION	:	POLYPROPYLENE

WIDTH 	:	MINIMUM 12 MM

THICKNESS 	:	MINIMUM 0.05 MM

BREAKING LOAD	:	80 KG/12 MM WIDTH (MIN.)

ELONGATION	:	MAX. 25%

-99-

ANNEXURE – XXIV

SPECIFICATIONS DETAILS

PRODUCT : DEHYDRATED GARLIC POWDER

UNIT POUCHES AND BULK PACK FOR UNIT POUCHES

I.	UNIT POUCH

MATERIAL				THICKNESS

1.	PET / AL. FOIL / LDPE	:	       12         /     9            /     37.5

LAMINATE                                        MICRON  /  MICRON /  MICRON

2.	MET-PET/LDPE		:	    12          /    37.5

MICRON  /    MICRON

The laminates shall be either extrusion laminated or in case of adhesive lamination, food grade odour free adhesives should be used

DIMENSIONS  (EXTERNAL)

(mm)

100 GRAMS			:	150 X 130 X 10

200 GRAMS			:	170 X 140 X 10

Performance:

Bond Strength (g/15 mm width)	 Min.

(In case of laminates only)				:	250.00

Heat Seal Strength (kg/15 mm width) Min.		:	2.00

Seal width (mm/Min.) 					:	5.00

Oxygen Transmission Rate (cc/m2/24 Hrs.) Max.

At 1 atm pressure 						:	3.00

Water Vapour Transmission Rate (g/m2/24 Hrs.)Max.

At 38  1C & 90  2% RH				:	1.00

- 99 A -

Note:

1.	While the above are indicative types of laminates that are recommended, any other flexible laminates or films comparable to the barrier and strength properties of the above types of laminates would also be acceptable.

2.       The laminate/co-extruded film could be transparent/ opaque/

coloured as per the requirement of the buyer

3.   	All the materials used shall be of food grade quality and pass the prescribed Indian Standards/Standards if any and as demanded by     the buyers.

4.	Labelling and printing requirements shall conform to as prescribed by the buyer.

5.	General requirements:

a)	The pouches shall be clean and without specs.

b)	The pouches shall be three sides or four sides sealed.

c)	The seal width shall be minimum 5 mm, particularly the

top seal. (For filled pouches)

d)	The pouches shall be either reverse or surface printed. In

the case of surface print, the print shall be compatible with the product and shall not smudge. The print shall be legible and easy to read.

e)	The pouch shall not fail either on the body or along the

seals when subjected to normal drop tests.

f)	All the pouches should be flushed with Nitrogen gas at

the time of  sealing to retain the freshness of the product.

-100-

# II.	BULK BOX

BOX FOR 6 KG AND 8 KG.

MATERIAL OF CONSTRUCTION	:	CORRUGATED FIBRE BOARD

SYLE OF BOX 	:	RSC

NO. OF PLIES	:	FIVE

TYPE OF FLUTE	:	B/B (NARROW/NARROW)

POSITION OF FLUTES	:	VERTICAL

DIMENSIONS (EXTERNAL) MM

6 KG BULK, 100 GMS UNIT PACK	:	635 X 430 X 100

8 KG BULK, 100 GMS UNIT PACK	:	570 X 510 X 120

6 KG BULK, 200 GMS UNIT PACK	:	415 X 330 X 145

8 KG BULK, 200 GMS UNIT PACK	:	550 X 330 X 145

GRAMMAGE	:	200/140/140/140/140

BURSTING STRENGTH (kg/cm2)	:	MINIMUM 14.00

COMPRESSION STRENGTH (kg)	:	MINIMUM 550.00

COBB VALUE (g/m2) (30 Min.)	:	MAXIMUM 155.00

NO. OF PIECES PER BOX	:	NOT MORE THAN TWO

MANUFACTURERS JOINT	:	BY STAPLE PINS

OTHER DETAILS AS PER IS:2771-1977 (PART-I)

-101-

# SEPARATOR PLATES

MATERIAL OF CONSTRUCTION	:	CORRUGATED FIBRE BOARD

NO. OF PLIES	:	THREE

TYPE OF FLUTE	:	B (NARROW)

GRAMMAGE (g/m2)	:	100/100/100

BURSTING STRENGTH (KG/CM.SQ)	:	MINIMUM 5.00

# PRESSURE SENSITIVE TAPE

MATERIAL	:	BOPP OR PVC

THICKNESS 	:	20 MICRONS

WIDTH	:	50 MM

ADHESIVE PROPERTY TO PASS AS PER IS:3676-1986

FOR FURTHER SPECIFICATION DETAILS REFER IS: 2880-1978

## REINFORCEMENT STRAP

MATERIAL OF CONSTRUCTION	:	POLYPROPYLENE

WIDTH 	:	MINIMUM 12 MM

THICKNESS 	:	MINIMUM 0.05 MM

BREAKING LOAD	:	80 KG/12 MM WIDTH (MIN.)

ELONGATION	:	MAX. 25%

-102-

ANNEXURE – XXV

SPECIFICATIONS DETAILS

PRODUCT : AFD CORN

UNIT POUCHES AND BULK PACK FOR UNIT POUCHES

I.	UNIT POUCH

MATERIAL				THICKNESS

1.	PET / AL. FOIL / LDPE	:	       12         /     9            /     37.5

LAMINATE                                        MICRON  /  MICRON /  MICRON

2.	MET-PET/LDPE		:	    12          /    37.5

MICRON  /    MICRON

The laminates shall be either extrusion laminated or in case of adhesive lamination, food grade odour free adhesives should be used

DIMENSIONS  (EXTERNAL)

(mm)

50 GRAMS			:	160 X 135 X 10

100 GRAMS			:	200 X 145 X 10

Performance:

Bond Strength (g/15 mm width)	 Min.

(In case of laminates only)				:	250.00

Heat Seal Strength (kg/15 mm width) Min.		:	2.00

Seal width (mm/Min.) 					:	5.00

Oxygen Transmission Rate (cc/m2/24 Hrs.) Max.

At 1 atm pressure 						:	3.00

Water Vapour Transmission Rate (g/m2/24 Hrs.)Max.

At 38  1C & 90  2% RH				:	1.00

- 102 A -

Note:

1.	While the above are indicative types of laminates that are recommended, any other flexible laminates or films comparable to the barrier and strength properties of the above types of laminates would also be acceptable.

2.       The laminate/co-extruded film could be transparent/ opaque/

coloured as per the requirement of the buyer

3.   	All the materials used shall be of food grade quality and pass the prescribed Indian Standards/Standards if any and as demanded by     the buyers.

4.	Labelling and printing requirements shall conform to as prescribed by the buyer.

5.	General requirements:

a)	The pouches shall be clean and without specs.

b)	The pouches shall be three sides or four sides sealed.

c)	The seal width shall be minimum 5 mm, particularly the

top seal. (For filled pouches)

d)	The pouches shall be either reverse or surface printed. In

the case of surface print, the print shall be compatible with the product and shall not smudge. The print shall be legible and easy to read.

e)	The pouch shall not fail either on the body or along the

seals when subjected to normal drop tests.

f)	All the pouches should be flushed with Nitrogen gas at

the time of  sealing to retain the freshness of the product.

-103-

# II.	BULK BOX

BOX  FOR 4 KG AND 6 KG.

MATERIAL OF CONSTRUCTION	:	CORRUGATED FIBRE BOARD

SYLE OF BOX 	:	RSC

NO. OF PLIES	:	FIVE

TYPE OF FLUTE	:	B/B (NARROW/NARROW)

POSITION OF FLUTES	:	VERTICAL

DIMENSIONS (EXTERNAL) MM

4 KG BULK, 50 GMS UNIT PACK	:	610 X 530 X 95

6 KG BULK, 50 GMS UNIT PACK	:	610 X 400 X 170

4 KG BULK, 100 GMS UNIT PACK	:	710 X 390 X 100

6 KG BULK, 100 GMS UNIT PACK	:	710 X 390 X 140

GRAMMAGE (g/m2)	:	200/140/140/140/140

BURSTING STRENGTH (KG/CM.SQ)	:	MINIMUM 14.00

COMPRESSION STRENGTH (KG)	:	MINIMUM 550.00

COBB VALUE (G/M.SQ) (30 MIN.)	:	MAXIMUM 155.00

NO. OF PIECES PER BOX	:	NOT MORE THAN TWO

MANUFACTURERS JOINT	:	BY STAPLE PINS

OTHER DETAILS AS PER IS:2771-1977 (PART-I)

-104-

# SEPARATOR PLATES

MATERIAL OF CONSTRUCTION	:	CORRUGATED FIBRE BOARD

NO. OF PLIES	:	THREE

TYPE OF FLUTE	:	B (NARROW)

GRAMMAGE (g/m2)	:	100/100/100

BURSTING STRENGTH (KG/CM.SQ)	:	MINIMUM 5.00

# PRESSURE SENSITIVE TAPE

MATERIAL	:	BOPP OR PVC

THICKNESS 	:	20 MICRONS

WIDTH	:	50 MM

ADHESIVE PROPERTY TO PASS AS PER IS:3676-1986

FOR FURTHER SPECIFICATION DETAILS REFER IS: 2880-1978

## REINFORCEMENT STRAP

MATERIAL OF CONSTRUCTION	:	POLYPROPYLENE

WIDTH 	:	MINIMUM 12 MM

THICKNESS 	:	MINIMUM 0.05 MM

BREAKING LOAD	:	80 KG/12 MM WIDTH (MIN.)

ELONGATION	:	MAX. 25%

-105-

#### ANNEXURE-XXVI

### SPECIFICATION DETAILS FOR MOULDED VACCUM PACK

- BARRIER BAG FOR 12.5 KG.ONION FLAKES/25 KG.GARLIC POWDER & 10 KG. AFD CORN

MATERIAL OF CONSTRUCTION				THICKNESS

- Laminates

PET/AL. FOIL/LDPE         :  12 MICRON/12-15 MICRON/90 MICRON (Min.)

- Co-extruded Film

LDPE-Tie-NYLON-Tie-LDPE	:	110 MICRON (Min.)

SEAL WIDTH	:	6 MM (Min.)

WATER VAPOUR TRANSMISSION RATE

g/m2/24 Hrs. (at 38C & 90% RH)

(Maximum)	:	1.00

OXYGEN TRANSMISSION RATE

cc/m2/24 Hrs. at 1 atm. (MAX.)	:	30.00

-105A-

Note:

1.	While the above are indicative types of laminates that are recommended, any other flexible laminates or films comparable to the barrier and strength properties of the above types of laminates would also be acceptable.

2.       The laminate/co-extruded film could be transparent/ opaque/

coloured as per the requirement of the buyer

3.   	All the materials used shall be of food grade quality and pass the prescribed Indian Standards/Standards if any and as demanded by     the buyers.

4.	Labelling and printing requirements shall conform to as prescribed by the buyer.

- General requirements of Annexure-XXIII also apply in this

case.

-106-

# BULK BOX

###### OUTER BOX FOR 2 PACKS (12.5 KG X 2) OF ONION FLAKES/1 PACK (25 KG.)  OF GARLIC POWDER.

MATERIAL OF CONSTRUCTION	:	5-PLY CORRUGATED FIBRE

BOARD

SYLE OF BOX 	:	RSC

DIMENSION OF BOX (EXTERNAL) MM 	:    390 X 270 X 285 (ONION FLAKES)

:    440 X 230 X 305 (GARLIC POWDER)

TYPE OF FLUTE	:	B/B (NARROW/NARROW)

DIRECTION OF FLUTES	:	VERTICAL

GRAMMAGE (Indicative)	:	180/180/180/180/180

BURSTING STRENGTH (kg/cm2)	:	MINIMUM 18.00

BURST FACTOR 	:	24.00

COMPRESSION STRENGTH (kg)	:	MINIMUM 550.00

COBB VALUE (g/m2) (30 Min.)	:	MAXIMUM 120.00

NO. OF PIECES PER BOX	:	NOT MORE THAN TWO

MANUFACTURERS JOINT	:	BY STAPLE PINS

TYPE OF ADHESIVE	:	Starch or any other suitable

Neutral adhesive.

-107-

# BULK BOX

B. OUTER BOX FOR 1 PACK OF 10 KGS. AFD CORN.

MATERIAL OF CONSTRUCTION	:	3-PLY CORRUGATED FIBRE

BOARD

SYLE OF BOX 	:	RSC

DIMENSION OF BOX (EXTERNAL) MM 	:          495 X 220 X 290

TYPE OF FLUTE	:	B/B (NARROW/NARROW)

DIRECTION OF FLUTES	:	VERTICAL

GRAMMAGE	:	180/140/180

BURSTING STRENGTH (kg/cm2)	:	MINIMUM 11.00

BURST FACTOR 	:	22-24

COMPRESSION STRENGTH (kg)	:	MINIMUM 300.00

COBB VALUE (g/m2) (30 Min.)	:	MAXIMUM 120.00

NO. OF PIECES PER BOX	:	NOT MORE THAN TWO

TYPE OF ADHESIVE	:	Starch or any other suitable

Neutral adhesive.